# VISUALISATIONS

By Anicet A 3/23/2026

Course: CIVE202 – Spring 2026
Instructor: Dr. Kaycie Lane

Table of Contents

Introduction and Background	3

Scope of Work and Objectives	3

Data Visualization	4

Bar Chart	4

Histogram	5

Boxplot	7

Time Series	9

Intelligent Driver Model (IDM)	11

Methodology	13

Conclusion	14

Sources	15

## Introduction and Background

Transportation is an economic development pillar because it facilitates the movement of goods, services, and human beings and promotes regional and international integration (Mohith et al., 2025). To better understand how people travel and how vehicles behave in real traffic environments, federal agencies collect large-scale datasets that capture both household travel patterns and detailed vehicle trajectory information. Two of the most influential datasets in this field are the National Household Travel Survey (NHTS) and the Next Generation Simulation (NGSIM) dataset. Together, these resources provide a comprehensive view of transportation usage, driver behavior, and the factors that shape system performance.

For this project, both datasets were provided to the firm’s transportation planning group to support an upcoming meeting focused on data-driven decision-making. As the Engineering Intern with the strongest background in Python programming, I was tasked with organizing, analyzing, and visualizing key elements of the data.

## Scope of Work and Objectives

The objective is to produce clear, meaningful graphics that highlight important trends and behaviors relevant to transportation planning. These visualizations include a bar chart comparing categories within a selected variable, a histogram illustrating the distribution of a variable, a boxplot showing variation across categories, and at least two time-series plots that examine driving behavior over time. The specific variables used are not predetermined; instead, they are selected based on their relevance to U.S. travel patterns and their ability to provide insight into the datasets. Each visualization uses a different variable, incorporates proper titles and labels, and demonstrates at least one customized feature using libraries such as Matplotlib or Seaborn.

In addition to the visual analysis, the project includes a simulation study using the Intelligent Driver Model (IDM). This simulation provides a theoretical representation of vehicle behavior and trajectory patterns under varying traffic conditions, offering a useful comparison to the real-world driving data captured in NGSIM.

Finally, the project requires detailed documentation of the time spent on each task. This timesheet will help the firm evaluate the efficiency of performing data analysis in Python compared with traditional Excel workflows. The results will inform future decisions about investing in Python training for Engineering Interns and assessing the potential time savings associated with adopting Python-based analytical methods.

## Data Visualization

### Bar Chart

Household Size Distribution

A bar chart illustrating the number of households by household size within the NHTS dataset. “Understanding the number of households by household size is essential in NHTS analysis because it directly supports the survey’s ability to measure travel behavior at the household level and identify demographic trends that influence transportation needs” (Oak Ridge National Laboratory, 2025). This visualization provides an essential demographic baseline for the analysis, as household size is one of the strongest predictors of travel behavior. Larger households typically generate more trips, own more vehicles, and exhibit different travel patterns than smaller households. Understanding the distribution of household sizes in the dataset helps the transportation planning group assess the representativeness of the sample and interpret subsequent analyses such as vehicle ownership, trip-making behavior, and mode choice with appropriate context.

Figure 1:Number of households by household size.

This bar chart illustrates how households in the dataset are distributed by household size. The results show that two-person households are by far the most common, followed by one-person households. Household sizes of three and four people also appear frequently, though in smaller numbers. Beyond four people, the counts drop sharply, with large households (seven or more members) representing only a very small portion of the sample.

This distribution provides an important demographic baseline for the project, helping contextualize later analyses of vehicle ownership, travel behavior, and driving patterns. Understanding the prevalence of different household sizes allows the transportation planning group to better interpret how household composition influences transportation needs and system usage.

## Histogram

Vehicle counts by vehicles type

A histogram of vehicle types in the NHTS dataset is more than a simple count of categories. It provides a foundational view of the composition of the U.S. household vehicle fleet, which is essential for understanding travel behavior, infrastructure needs, and policy planning. “The NHTS provides detailed information on household vehicles because vehicle characteristics such as type, age, and fuel technology directly influence travel behavior, trip-making patterns, and the overall performance of the household vehicle fleet.” (U.S. Department of Transportation, Federal Highway Administration, 2024). Different vehicles support different kinds of trips. For example:

- SUVs and vans are often used for family travel, group trips, and longer-distance travel.

- Small cars are more common for commuting or short urban trips.

- Pickup trucks are associated with work-related travel or rural households.

Because of these differences, the type of vehicle a household owns shapes the purpose, distance, and frequency of trips.

Figure 2: Vehicle count by vehicle types

The histogram illustrates how frequently each vehicle category appears in the NHTS sample. Categories such as SUVs, automobiles/station wagons, pickup trucks, and vans dominate the distribution, while motorcycles, RVs, and other specialty vehicles appear far less often. This pattern reflects real-world ownership trends in the United States, where light duty vehicles especially SUVs and sedans make up most household fleets.

## Boxplot

Vehicle Age by Vehicle type

A box plot illustrates a compact, visual summary of how old different types of vehicles are in the NHTS dataset, and it matters because vehicle age is directly tied to safety, emissions and maintenance costs. Understanding these patterns helps explain why households own the vehicles they do and how those vehicles shape national travel trends. “Vehicles on the road are getting even older… the aging fleet is expected to provide additional vehicles to the aftermarket for repairs and maintenance.” (Parekh & Campau, 2024). This quote supports the idea that older vehicles behave differently in the fleet. The box plot highlights which vehicle types tend to be older and therefore more likely to require maintenance or pose safety concerns.

Figure 3:Vehicle age by vehicle type

What the box plot shows:

Median age differences across vehicle types

Each box has a horizontal line inside it: the median. The line shows that:

- SUVs and automobiles tend to have younger median ages, meaning households replace or purchase these vehicles more frequently.

- RVs, other trucks, and motorcycles have older medians, suggesting they are kept longer and used less frequently.

This tells you which vehicles are part of the “active” household fleet versus those that are more specialized or occasional use.

Maximum and minimum ages

The whiskers show the full range of ages.

- Some categories (like RVs) reach 30–40 years.

- Most cars and SUVs stay below 20 years.

This reflects how long different vehicle types remain in the household fleet.

Spread of ages (IQR)

The height of each box represents the interquartile range (IQR) the middle 50% of vehicle ages.

- Automobiles and SUVs have tighter boxes, meaning their ages are more consistent.

- RVs and other trucks have very tall boxes, showing a wide range of ages.

This indicates that some vehicle categories have predictable replacement cycles, while others vary widely.

Outliers

Dots above the whiskers represent unusually old vehicles.

- RVs and “other trucks” show many outliers, meaning some households keep these vehicles for decades.

- Cars and SUVs have fewer extreme outliers.

Outliers help identify unusual ownership patterns.

## Time Series

Leaders and Followers Position over time.

A time-series plot is one of the most powerful ways to understand how two vehicles interact over time. In the graph, the blue lines show the leader’s position, and the orange lines show the follower’s position, both plotted against time. Because NGSIM records data every 0.1 seconds, the plot captures the continuous motion of each vehicle with high precision.

Figure 4: Leader and Follower position over time.

The upward trend in both lines reflects continuous forward movement. This is expected in freeway traffic and confirms that the dataset is capturing normal driving behavior rather than stops or unusual events. The orange lines generally run parallel to the blue lines. This indicates that the followers maintain a consistent spacing behind leaders, their speeds adjust to match the leader’s speed and the follower’s trajectory is influenced by the leader’s behavior. This is the core of car-following dynamics. Where the follower line slightly diverges from the leader line, you can see delayed reactions, slight over- or under-shooting in acceleration, micro-adjustments in spacing. Understanding these patterns matter because it connects trajectory data to real-world driving behavior and in Transportation engineering it is critical to study reaction time, instability in traffic flow and safe/Unsafe following distances.

Leaders and followers speed over time.

This series plot tracks how the leader and follower vehicles change their speeds over the same time interval. Each line represents the instantaneous speed recorded in the NGSIM dataset.

Figure 5:Leader vs Follower speed over time.

The lines rise and fall together, showing that traffic flow is not constant, drivers continuously adjust their speed in response to conditions ahead. The orange line (follower) closely mirrors the blue line (leader). This reflects real-world car-following behavior, when the leader speeds up, the follower speeds up shortly after, when the leader slows down, the follower also slows down. Small delays or mismatches show human reaction time.

# Intelligent Driver Model (IDM)

IDM Follower speed

Trajectory 16 was used to simulate the patterns of the follower speed based on the leaders. The green dashed line generally follows the orange line’s shape. That means your IDM parameters are reasonably calibrated, the simulated follower is responding to the leader in a way that resembles the real driver.

Figure 6: IDM Simulation of follower speed based on Leader

The figure above presents a comparison between the observed leader and follower vehicle speeds from the NGSIM dataset, and the follower speed generated by the Intelligent Driver Model (IDM). The leader’s speed profile serves as the input to the simulation, allowing the IDM to reproduce the follower’s car-following behavior over time. The observed follower speed closely aligns with the IDM-simulated speed throughout the 55-second period. Both profiles show similar acceleration and deceleration patterns, indicating that the calibrated IDM parameters capture the general responsiveness of the real driver. Minor deviations between the simulated and observed speeds highlight natural differences between human driving behavior and the model’s assumptions. For example, small lags or smoother transitions in the simulated profile reflect IDM’s tendency to produce more stable, less noisy responses than real-world drivers.

IDM Follower acceleration

The IDM captures the general acceleration trends. The green dashed line (IDM) follows the broad shape of the real follower’s acceleration. This means the model responds to the leader braking, leader acceleration and changes in spacing in a way that resembles human driving.

Figure 7:Acceleration comparison

Figure above presents a comparison of the instantaneous acceleration profiles for the leader vehicle, the observed follower vehicle, and the follower vehicle simulated using the Intelligent Driver Model (IDM). This comparison provides insight into how well the IDM reproduces the dynamic driving behavior observed in the NGSIM trajectory data. The leader and follower accelerations derived from NGSIM exhibit the expected variability associated with human driving. Both profiles contain short duration changes, rapid transitions between acceleration and deceleration. These characteristics reflect natural driver behavior, including reaction delays, small corrective inputs, and variability in comfort thresholds

## Methodology

The methodology for this project relied on a combination of structured in-class lab exercises, independent analysis, and selective AI-assisted debugging to produce the required visualizations and simulations. Most of the core code used throughout the project including data loading, cleaning, visualization setup, and the initial Intelligent Driver Model (IDM) framework was developed during class labs, which provided the foundational structure for working with the NHTS and NGSIM datasets. Building on this foundation, additional code was written to customize plots, refine analytical steps, and extend the IDM simulation to include both speed and acceleration comparisons. During implementation, certain portions of the code did not function as intended due to syntax errors, indexing issues, or logic inconsistencies; in these cases, AI assistance was used specifically to diagnose and correct the problematic lines while ensuring that the overall workflow, logic, and analytical decisions remained student-driven. All instances where AI contributed to code corrections or improvements are clearly identified in the accompanying Annotated Code Document to maintain full transparency. The final workflow included preparing and visualizing demographic and vehicle related variables from the NHTS dataset, generating time-series analyses from the NGSIM trajectory data, and implementing an IDM simulation using a forward-Euler integration approach to compare real and modeled driving behavior.

## Conclusion

This project demonstrates how Python-based data analysis and simulation can provide clear, actionable insights for transportation planning. By examining both the NHTS and NGSIM datasets, the analysis highlights the value of combining large-scale household travel data with high-resolution vehicle trajectory information to understand travel patterns and driver behavior. The visualizations created from the NHTS dataset establish an essential demographic and behavioral foundation. The bar chart of household size distribution shows how household composition influences travel demand, while the histogram of vehicle types reveals the makeup of the U.S. vehicle fleet and its implications for trip purposes and transportation needs. The boxplot of vehicle age further emphasizes differences in safety, maintenance, and usage patterns across vehicle categories. Together, these graphics provide a comprehensive overview of the factors that shape national travel behavior and support data-driven decision making. The NGSIM time-series analyses deepen this understanding by illustrating real-world car-following behavior at a fine temporal scale. The position and speed plots show how closely followers react to leaders, capturing the subtle dynamics of acceleration, deceleration, and spacing adjustments that occur in freeway traffic. These observations form the basis for evaluating the Intelligent Driver Model (IDM), a theoretical framework widely used in traffic flow and microsimulation studies. The IDM simulation results demonstrate that the model can reasonably replicate the follower’s speed and acceleration patterns when provided with the leader’s trajectory. The comparison between observed and simulated behavior shows that the IDM captures the essential structure of human driving—responding appropriately to changes in speed and spacing—while also revealing the smoother, more stable nature of model-based acceleration. These findings validate the IDM’s usefulness for representing car-following behavior and highlight opportunities for parameter refinement to better match real-world variability.

## Sources

Mohith, S., Mahesh, B., Kumar, M., & Sandhya, S. (2025). The Role of Transportation in Economic Development: A Comprehensive Analysis. RV College of Engineering.

Oak Ridge National Laboratory. (2025). How does number of households by household size help the NHTS? Retrieved from Bing search results.

U.S. Department of Transportation, Federal Highway Administration. (2024). Summary of Travel Trends: 2022 National Household Travel Survey (FHWA-HPL-24-009). Bureau of Transportation Statistics

Parekh, N., & Campau, T. (2024). Fuel for Thought: Average age of vehicles in the US hits new record. S&P Global Mobility.

OpenAI. (2025). ChatGPT [Large language model]. https://chat.openai.com/